| # REPBULIC OF RWANDA MINISTRY OF HEALTH …………. PROVINCE …………. DISTRICT …………. HOSPITAL P.O Box …………….. # Email: ……………… | Policy/Procedure Title: Personnel Protective Equipment (PPE) Use |  |  |
| --- | --- | --- | --- |
|  | Policy code/ Number: ES3-15 | # Effective Date: August 2018 | # Revision date: July 2020 |
|  | Department: Clinical Department | Applies to: all staff, patients, and visitors |  |

|  | Position | Names | Date& Signature |
| --- | --- | --- | --- |
| Responsible | Environmental Health Officer |  |  |
| Supervisor | Director of Medical Allied Health Sciences Services |  |  |
| Approval | Director General |  |  |

Purpose: To ensure hospital staff, patients, and visitors are protected from acquiring healthcare associated infections through the proper management of personal protective equipment (PPE).

Policy Statements:

- The hospital shall ensure that PPEs are availed in all relevant hospital areas for staff, patients, and visitors to control healthcare associated infections.

Definitions:

- Personal Protective Equipment (PPE): specialized clothing or equipment worn by an employee working in the health care service and any person in contact with contaminated or suspected area for protection against infection.

Equipment/Forms: Specific components of PPE, include:

- Gloves

- Face masks

- Gowns

- Plastic Apron

- Overalls

- Head cover

- Foot Protection

- Goggles

Procedures:

## Hand protection

Gloves

- Ensure gloves are stored in an appropriate storage location.

- Ensure the right gloves are availed for the right task.

- Remove all jewelry on hands, including a watch. Not only can they cause your glove to rip, they can harbor unwanted microorganisms. Also make sure that your fingernails are clean and short (no longer than ¼ inch in length) and no nail polish.

- Wash hands with liquid soap prior to wearing gloves to avoid transfer of microorganisms.

- Check gloves for good barrier properties to ensure that it is flexible, free from holes, breaches, and cracks. Gloves should be strong enough to prevent breakage during normal use.

- Change gloves and disinfect hands between tasks and procedures, or after contact with suspected material.

- Remove gloves promptly after use and dispose them in an appropriate receptacle.

- Wash hands at the end of all procedures to avoid transfer of microorganisms to other patients or environments.

Body Protection

Medical gown

- Wearing a clean gown is required to enter the theatre, neonatology, and sterilization rooms.

- Gown sleeves must cover the wrists.

- Put on gown before putting on gloves.

- Gloves must cover the gown cuffs.

- Do not wear a gown outside theatre, neonatology, and sterilization unless transporting contaminated items.

- If soiled, remove gown immediately.

- Take off gloves and perform hand hygiene before taking off gown.

- Place used, disposable gowns in regular waste container and perform hand hygiene.

- Place washable gowns in linen buckets dedicated to room and perform hand hygiene.

Paper gown

- Paper gown that has ties at the neck and at the waist must be worn to protect self from infectious client and also to protect from possible microorganisms.

- Both sets of ties need to be tied securely.

- Change between clients.

- Discard paper gown after use.

Plastic Apron

- Worn to reduce contact with blood, bodily secretions, excretions, disinfectants, and chemicals.

- Have ties at the neck and at the waist.

- Both sets of ties need to be tied up.

- Clean and store in a dry place between uses.

- Change apron each time a different patient is visited.

Overalls

- Wear when there is a risk of splashing from corrosive materials.

- Change if overalls become contaminated.

- Disposable overalls must be correctly discarded after use.

## Respiratory Protection

Face Mask

- Wear face mask in areas where droplet infection of the patient is a concern.

- Wear face masks when the worker has a cold.

- Cover the nose and mouth.

- Secure face mask correctly behind your ears with elastic fastenings.

- Face mask cannot be worn for extended periods.

## Eye Protection

## Safety Spectacles/ Goggles

- Wear when there is the risk of eye injury from splashing.

- May look like normal glasses or may have side shields.

- Cannot wear glasses underneath safety spectacles. Goggles have a larger area of coverage, may wear glasses underneath.

- Clean after use.

## Head Protection

## Hairnet/Cover

- Hair must be tied up and completely tucked into cover.

- Wear a hairnet to prevent hair contamination.

Foot Protection

## Shoe covers

- Wear to protect from contamination when entering an area of infection either clean or critical areas. Wear at all times to reduce contact with blood, bodily secretions, excretions, disinfectants, chemicals.

- Wear to prevent contamination from spreading.

- Cover shoe completely and tie securely.

- Discard after use.

References:

- Perry J and Jagger J. Occupational Co-Infection with HIV and HCV in Clinical Lab Via Blood Splash. Advances in Exposure Prevention 7. 2005.

- Siegel JD, Rhinehart E, et al. 2007 Guideline for Isolation Precautions: Preventing Transmission of Infectious Agents in Health Care Settings." Am J Infect Control. 35(10 Suppl 2): S65-164. 2007.

- Casey T. Top 10 OSHA Citations in the Healthcare Industry. LexisNexis Martindale-Hubbell. February 2012.  DI

Appendix: